ARCHITECTURAL AND ENGINEERING SPECIFICATION

# Industrial Fiber-Optic Transmission System

Basis of Design: KBC Networks ASFOM-FPA

| Product family: Industrial fiber-optic transmission equipment | Document status: Tender / design specification |
| --- | --- |
| Source data: specsheet-asfom_fpa_series-low-rev1401a.pdf | Prepared: 4 August 2026 |

# PART 1 - GENERAL

## 1.1 SUMMARY

Provide, install, configure where applicable, test and commission the industrial fiber-optic transmission system described in this specification, including the required interfaces, power connections, mounting hardware and compatible accessories.

## 1.2 BASIS OF DESIGN

Basis-of-design manufacturer and product: KBC Networks ASFOM-FPA. Source description: Audio Point-to-Point Transmission.

Equivalent products may be considered only when they meet or exceed every stated interface, performance, power, environmental and mounting requirement without changing the intended system design.

## 1.3 SUBMITTALS

Manufacturer technical data sheet identifying the exact model and selected hardware variant.

Interface or connection schedule identifying the electrical, optical, wireless, power and network connections.

Power calculation confirming input-supply compatibility and the total connected load where applicable.

Mounting, environmental and configuration details applicable to the proposed installation.

## 1.4 QUALITY ASSURANCE

Installation shall be performed by personnel familiar with industrial fiber-optic transmission, optical power budgets, connector and wavelength selection, signal interfacing and applicable electrical and network practices. Verify all product capability from the current manufacturer data sheet and selected part number.

# PART 2 - PRODUCTS

## 2.1 REQUIRED PRODUCT

The equipment shall be KBC Networks ASFOM-FPA, or an approved equivalent meeting all requirements below. It shall be supplied as audio point-to-point transmission.

## 2.2 PRODUCT DESCRIPTION

The KBC FPA series is part of the ASFOM product family and provides unique, user configurable solutions for audio signals. It supports transmission of up to 128 simplex or 64 duplex audio signals over either one multimode or singlemode optical fiber. Due to the ASFOM flexibility, duplex solutions can either be symmetrical or asymmetrical with respect to the signal quantities.

The plug-and-play design ensures ease of installation with no electrical or optical adjustment needed. LED indicators are provided to clearly show the operational status of the unit. Depending on system requirements, units are available in wall-mount, 1U or 2U 19” rack or 4U chassis card.

## 2.3 KEY FEATURES

Up to 128 simplex or 64 duplex audio signals

24 bit digitally-encoded audio transmission

Multimode and singlemode

Wall-mount, rack or chassis configurations

Unique Configurator software package

No fixed configuration

## 2.4 SOURCE-VERIFIED TECHNICAL REQUIREMENTS

| Requirement | Specified performance |
| --- | --- |
| Audio - No. of Channels Simplex | 128 max |
| Environmental - Operating Temperature | -40°~+74°C / -40°~+165°F |
| Audio - Duplex | 64 max |
| Environmental - Storage Temperature | -40°~+74°C / -40°~+165°F |
| Audio - Bandwidth | 10 Hz ~ 20 kHz |
| Environmental - Operating Humidity | 0 to 95% non-condensing |
| Audio - Input/Output Levels | +/- 9dBm, typical 0dBm |
| Environmental - Mean Time Between Failure (MTBF) | >100,000 Hours |
| Audio - Input Impedance | 600Ω unbalanced |
| Audio - Output Impedance | 20kΩ unbalanced or balanced |
| Mechanical - Dimensions (Wall-mount L x W x H)(5) | 221mm x 155mm x 36mm (8.7” x 6.1” x 1.42”) |
| Audio - Digital Encoding | 24 bits (AES-3) |
| Audio - Sampling Rate | 192 kHz |
| Mechanical - Dimensions (Rack) | 1U 19”, 1U |
| Audio - Signal to Noise Ratio | ≥80dB |
| Mechanical - 2U | 19”, 2U (2) |
| Audio - Total Harmonic Distortion | ≤0.01% |
| Mechanical - Number of 4U Chassis Card Slots | Depends on configuration |
| Optical - Wavelength | 1310nm, 1550nm |
| Optical - Fiber | Multimode or singlemode |
| Optical - Number of Fibers | 1 |
| Connectors - Audio | 6-way screw block terminal |
| Connectors - Fiber | ST, SC or FC (ST fitted as standard) |
| Power - Power Input Wall-mount | +12-24 Vac/dc |
| Connectors - Power(3) | Wall-mount Screw block terminal |
| Power - Power Supply for Wall-mount(1) | Input: 100 ~ 240 Vac Output: +12 Vdc |
| Connectors - Rack | IEC |
| Power - Rack Module(1) | Input: 100 ~ 240 Vac @ 2A |
| Power - Chassis Card | Supplied from FR3 or FR4 PSU |

The installer shall verify the selected part number, accessories and all project-specific operating limits against the current manufacturer data sheet before procurement and installation.

## 2.5 SOURCE VERIFICATION NOTE

The requirements in this document have been prepared from specsheet-asfom_fpa_series-low-rev1401a.pdf. Any configurable channel count, optical module, antenna, power supply, plug, mounting option, battery, solar array, accessory or regional variant shall be selected and confirmed separately where the source sheet presents multiple possibilities.

# PART 3 - EXECUTION

## 3.1 EXAMINATION

Confirm that the proposed equipment and selected variant match the approved design, required interfaces and intended operating environment.

Confirm that all input-power, cable, fiber, antenna, network and mounting conditions are within the manufacturer limits.

Confirm that all required accessories, transceivers, power supplies, batteries, antennas, mounting components and interconnection items are available and compatible.

Do not begin installation until conflicts between the design, site conditions and manufacturer requirements have been resolved.

## 3.2 INSTALLATION

Confirm signal type, channel count, direction, connector, wavelength, fiber type and link distance for every optical path.

Confirm compatible transmitter, receiver, card, chassis and power-supply arrangements before installation.

Protect and route fiber in accordance with the approved drawings, maintaining bend radius and connector cleanliness.

Label all electrical interfaces, optical links, fibers, cards, chassis positions and power circuits.

## 3.3 CONFIGURATION AND COMMISSIONING

Verify operation of every electrical signal and optical path included in the system.

Measure or otherwise confirm optical link performance and verify that the installed loss is within the approved budget.

Demonstrate alarm, redundancy and recovery functions where supported.

Provide final channel schedules, optical test results and installed model/serial information.